REUSE MANUAL

GEOID

10xxxxxx.1

Implementation

# Table of Contents

Table of Contents	i

SECTION 1.  Introduction	1

1.1	Purpose of the reuse Manual	1

1.2	PURPOSE OF THE REUSABLE SOFTWARE COMPONENT	1

1.3	GENERAL INFORMATION	2

1.3.1	POINT OF CONTACT	2

1.3.2	CERTIFICATION LEVEL	2

1.3.3	LEGAL RESTRICTIONS	2

Section 2.  INSTALLATION	3

2.1  PARTIAL REUSE	3

2.2  MODIFICATIONS	3

Section 3.  ENVIRONMENT	4

3.1	HARDWARE	4

3.1.1	DEVELOPMENT	4

3.1.2	TARGET	4

3.2	SOFTWARE	4

3.2.1	OPERATING SYSTEM	4

3.2.2	COMPILERS	4

3.3	ASSUMPTIONS AND PERFORMANCE LIMITATIONS	5

Section 4.  Global RSC Environment	6

4.1	TYPES	6

4.2	CONSTANTS	6

4.3	VARIABLES	6

4.4	INCLUDE FILES	6

4.5	dependencies	6

SECTION  5.  FUNCTIONS	7

5.1	INITIALIZE_GEOID	7

5.1.1	DESCRIPTION	7

5.1.2	INTERFACES AND EXAMPLES	7

5.1.3	DECLARATIONS	7

5.1.4	DEPENDENCIES	8

5.1.5	ERROR HANDLING	8

5.2	READ_FLOAT	8

5.2.1	DESCRIPTION	8

5.2.2	INTERFACES AND EXAMPLES	8

5.2.3	DECLARATIONS	8

5.2.4	DEPENDENCIES	9

5.2.5	ERROR HANDLING	9

5.3	INITIALIZE_EGM96_GEOID	9

5.3.1	DESCRIPTION	9

5.3.3	DECLARATIONS	9

5.3.4	DEPENDENCIES	10

5.3.5	ERROR hANDLING	10

5.4	INITIALIZE_EGM84_GEOID	10

5.4.1	DESCRIPTION	10

5.4.3	DECLARATIONS	11

5.4.4	DEPENDENCIES	11

5.4.5	ERROR hANDLING	11

5.5	BL_INTERPOLATE	11

5.5.1	DESCRIPTION	11

5.5.3 DECLARATIONS	12

5.5.4	DEPENDENCIES	12

5.5.5	ERROR hANDLING	12

5.6	NS_INTERPOLATE	13

5.6.1	DESCRIPTION	13

5.6.3 DECLARATIONS	13

5.6.4	DEPENDENCIES	13

5.6.5	ERROR hANDLING	14

5.7	Convert_ELLIPSOID_To_GEOID_HEIGHT	14

5.7.1	DESCRIPTION	14

5.7.2	INTERFACES AND EXAMPLES	14

5.7.3	DECLARATIONS	15

5.7.4	DEPENDENCIES	15

5.7.5	ERROR HANDLING	15

5.8	Convert_ELLIPSOID_To_MSL_EGM96_VG_NS_HEIGHT	15

5.8.1	DESCRIPTION	15

5.8.2	INTERFACES AND EXAMPLES	16

5.8.3	DECLARATIONS	16

5.8.4	DEPENDENCIES	17

5.8.5	ERROR HANDLING	17

5.9	Convert_ELLIPSOID_To_MSL_EGM84_10D_BL_HEIGHT	17

5.9.1	DESCRIPTION	17

5.9.2	INTERFACES AND EXAMPLES	17

5.9.3	DECLARATIONS	18

5.9.4	DEPENDENCIES	18

5.9.5	ERROR HANDLING	18

5.10	Convert_ELLIPSOID_To_MSL_EGM84_10D_NS_HEIGHT	19

5.10.1	DESCRIPTION	19

5.10.2	INTERFACES AND EXAMPLES	19

5.10.3	DECLARATIONS	20

5.10.4	DEPENDENCIES	20

5.10.5	ERROR HANDLING	20

5.11	Convert_GEOID_TO_ELLIPSOID_hEIGHT	20

5.11.1	DESCRIPTION	20

5.11.3	DECLARATIONS	21

5.11.4	DEPENDENCIES	22

5.11.5	ERROR HANDLING	22

5.12	Convert_MSL_EGM96_VG_NS_TO_ELLIPSOID_hEIGHT	22

5.12.1	DESCRIPTION	22

5.12.3	DECLARATIONS	23

5.12.4	DEPENDENCIES	23

5.12.5	ERROR HANDLING	23

5.13	Convert_MSL_EGM84_10D_BL_TO_ELLIPSOID_hEIGHT	24

5.13.1	DESCRIPTION	24

5.13.3	DECLARATIONS	25

5.13.4	DEPENDENCIES	25

5.13.5	ERROR HANDLING	25

5.14	Convert_MSL_EGM84_10D_NS_TO_ELLIPSOID_hEIGHT	25

5.14.1	DESCRIPTION	25

5.14.3	DECLARATIONS	26

5.14.4	DEPENDENCIES	27

5.14.5	ERROR HANDLING	27

APPENDIX A  STRUCTURE/DEPENDENCY DIAGRAMS	28

APPENDIX B  DEFINITIONS/GLOSSARY	29

APPENDIX C  REFERENCES	30

# SECTION 1.  Introduction

## 1.1	Purpose of the reuse Manual

This document describes the characteristics of the GEOID reusable software component and provides instructions on its installation and operation.  The manual is a self-contained reference for the software engineer intending to incorporate the component in another software system.  This manual was written with the assumption that the user has a basic working knowledge of C and is familiar with fundamental C concepts and terminology.

## 1.2	PURPOSE OF THE REUSABLE SOFTWARE COMPONENT

The purpose of GEOID is to provide a reusable software component that supports the following coordinate conversions:

- WGS 84 Ellipsoid height (meters) to EGM 96 Geoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 15-minute geoid separation grid and a bilinear interpolation method,

- WGS 84 Ellipsoid height (meters) to EGM 96 Geoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a variable resolution grid separation grid and a natural spline interpolation method,

- WGS 84 Ellipsoid height (meters) to EGM 84 Geoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 10-degree geoid separation grid and a bilinear interpolation method,

- WGS 84 Ellipsoid height (meters) to EGM 84 Geoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 10-degree geoid separation grid and a natural spline interpolation method,

- EGM 96 Geoid height (meters) to WGS 84 Ellipsoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 15-minute geoid separation grid and a bilinear interpolation method,

- EGM 96 Geoid height (meters) to WGS 84 Ellipsoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a variable resolution grid separation grid and a natural spline interpolation method,

- EGM 84 Geoid height (meters) to WGS 84 Ellipsoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 10-degree geoid separation grid and a bilinear interpolation method, and

- EGM 84 Geoid height (meters) to WGS 84 Ellipsoid height (meters) at the specified geodetic coordinates (latitude and longitude in radians), using a 10-degree geoid separation grid and a natural spline interpolation method.

A particular variation of the Geoid conversion is specified in terms of the following parameters:

- Longitude – Longitude (in radians) at the height conversion point,

- Latitude – Latitude (in radians) at the height conversion point.

## 1.3	GENERAL INFORMATION

### 1.3.1	POINT OF CONTACT

U.S. Army Topographic Engineering Center (USATEC)

Geospatial Informaton Division (GID)

ATTN:  CETEC-GD-A (Dan Specht)

7701 Telegraph Road

Alexandria, VA  22315-3864

Dan Specht		(703) 428 - 6761	Project Manager

### 1.3.2	CERTIFICATION LEVEL

This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

### 1.3.3	LEGAL RESTRICTIONS

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# Section 2.  INSTALLATION

The following is a list of the files that make up the GEOID component:

Source Code Files:

geoid.c

Header Files :

geoid.h

Data Files :

egm96.grd

egm84.grd

The compilation instructions for the GEOID component are as follows:

DOS Makefile (Uses Microsoft C):

cl /nologo /W3 /FR /G2 /DNDEBUG /Gs /Ox /AM /D_DOS /c geoid.c

UNIX Makefile (Uses gcc compiler):

cc -g -O -ansi -Wall -c geoid.c

The compilation order of the GEOID component relative to other components is unconstrained.

## 2.1  PARTIAL REUSE

The GEOID component does not allow for partial reuse.

## 2.2  MODIFICATIONS

The GEOID component does not permit modifications.

# Section 3.  ENVIRONMENT

This section provides details on the environment under which GEOID was developed, tested, and executed.

## 3.1	HARDWARE

### 3.1.1	DEVELOPMENT

The following is a list of hardware configurations under which GEOID was developed and tested.

- SUN SparcStation 20

- IBM compatible Pentium PC

### 3.1.2	TARGET

The following is a list of hardware configurations under which GEOID was executed.

- SUN SparcStation 20

- IBM compatible Pentium PC

## 3.2	SOFTWARE

### 3.2.1	OPERATING SYSTEM

The following is a list of operating systems under which GEOID was executed and tested.

- Solaris 2.7

- Windows 2000/NT/XP

### 3.2.2	COMPILERS

The following is a list of compilers on which GEOID was compiled successfully.

- GCC version 2.95.2

- Microsoft Visual C++ version 6

## 3.3	ASSUMPTIONS AND PERFORMANCE LIMITATIONS

There are no hardware or environment constraints.  There are no limitations.

This RSC is written in ANSI C.

# Section 4.  Global RSC Environment

## 4.1	TYPES

Not applicable.

## 4.2	CONSTANTS

The following is a list of significant visible constants declared globally in GEOID with their descriptions.

GEOID_NO_ERROR 	: No errors occurred in function

GEOID_FILE_OPEN_ERROR	: Geoid file opening error

GEOID_INITIALIZE_ERROR	: Geoid separation database can not initialize

GEOID_LAT_ERROR	: Latitude out of valid range (-90 to 90 degrees)

GEOID_LON_ERROR	: Longitude out of valid range (-180 to 360 degrees)

## 4.3	VARIABLES

The following is a list of significant global variables declared in GEOID with their descriptions.

/* Array of EGM96 elevations */

float egm96_height_grid[EGM96_ELEVATIONS];

/* Array of EGM84 elevations */

float egm84_height_grid[EGM84_ELEVATIONS];

FILE* geoid_height_file : A file of Geoid separation data

## 4.4	INCLUDE FILES

stdio.h                         : Used for file parsing

geoid.h	: Error codes and prototype error checking

## 4.5	dependencies

None, other than the standard ANSI C math and string-handling libraries.

# SECTION  5.  FUNCTIONS

## 5.1	INITIALIZE_GEOID

### 5.1.1	DESCRIPTION

This function reads Geoid separation data from the files named "egm96.grd" and “egm84.grd”, which must be located in the directory specified by the value of the environment variable "GEOTRANS_DATA", if defined, or else in the current directory.   If the files can not be found or accessed, an error code of GEOID_FILE_OPEN_ERROR is returned.  If the separation files are incomplete or improperly formatted, an error code of GEOID_INITIALIZE_ERROR is returned, otherwise GEOID_NO_ERROR is returned.

### 5.1.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Initialize_Geoid( )

Example:

- status = Initialize_Geoid ()

Inputs:

- None

Outputs:

- None

### 5.1.3	DECLARATIONS

#### 5.1.3.1	TYPES

Not applicable.

#### 5.1.3.2	CONSTANTS

Not applicable.

#### 5.1.3.3	VARIABLES

Not applicable.

### 5.1.4	DEPENDENCIES

- Initialize_EGM96_Geoid – used to read data from egm96.grd file

- Initialize_EGM84_Geoid – used to read data from egm84.grd file

### 5.1.5	ERROR HANDLING

This function returns the following status codes:

GEOID_FILE_OPEN_ERROR		: Geoid file opening error

GEOID_INITIALIZE_ERROR		: Geoid separation database can not initialize

## 5.2	READ_FLOAT

### 5.2.1	DESCRIPTION

This private function returns the geoid height read from a binary geoid file. 4 bytes are read from the file and, if necessary, the bytes are swapped.

### 5.2.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- float read_float ( int *num_read )

- num_read	Number of heights read from file.

### 5.2.3	DECLARATIONS

#### 5.2.3.1	TYPES

Not applicable.

#### 5.2.3.2	CONSTANTS

Not applicable.

#### 5.2.3.3	VARIABLES

Not applicable.

### 5.2.4	DEPENDENCIES

None.

### ERROR HANDLING

None.

## 5.3	INITIALIZE_EGM96_GEOID

### 5.3.1	DESCRIPTION

This private function reads Geoid separation data from the file named "egm96.grd”, which must be located in the directory specified by the value of the environment variable "GEOTRANS_DATA", if defined, or else in the current directory.   If the file can not be found or accessed, an error code of GEOID_FILE_OPEN_ERROR is returned.  If the separation file is incomplete or improperly formatted, an error code of GEOID_INITIALIZE_ERROR is returned, otherwise GEOID_NO_ERROR is returned.

5.3.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Initialize_EGM96_Geoid ()

### 5.3.3	DECLARATIONS

#### 5.3.3.1	TYPES

Not applicable.

#### 5.3.3.2	CONSTANTS

Not applicable.

#### 5.3.3.3	VARIABLES

Not applicable.

### 5.3.4	DEPENDENCIES

- read_float – used to read data from the binary geoid height file.

### ERROR hANDLING

This function returns the following status codes:

GEOID_FILE_OPEN_ERROR		: Geoid file opening error

GEOID_INITIALIZE_ERROR		: Geoid separation database can not initialize

## 5.4	INITIALIZE_EGM84_GEOID

### 5.4.1	DESCRIPTION

This private function reads Geoid separation data from the file named "egm84.grd”, which must be located in the directory specified by the value of the environment variable "GEOTRANS_DATA", if defined, or else in the current directory.   If the file can not be found or accessed, an error code of GEOID_FILE_OPEN_ERROR is returned.  If the separation file is incomplete or improperly formatted, an error code of GEOID_INITIALIZE_ERROR is returned, otherwise GEOID_NO_ERROR is returned.

5.4.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Initialize_EGM84_Geoid ()

### 5.4.3	DECLARATIONS

#### 5.4.3.1	TYPES

Not applicable.

#### 5.4.3.2	CONSTANTS

Not applicable.

#### 5.4.3.3	VARIABLES

Not applicable.

### 5.4.4	DEPENDENCIES

- read_float – used to read data from binary geoid height file.

### ERROR hANDLING

This function returns the following status codes:

GEOID_FILE_OPEN_ERROR		: Geoid file opening error

GEOID_INITIALIZE_ERROR		: Geoid separation database can not initialize

## 5.5	BL_INTERPOLATE

### 5.5.1	DESCRIPTION

This private function returns the height of the geoid above or below the WGS 84 ellipsoid, at the specified geodetic coordinates, using a grid of height adjustments and a bilinear interpolation method.

5.5.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long BL_Interpolate( double latitude,

- double longitude,

- double scale_factor,

- int num_cols,

- int num_rows,

- float height_buffer[],

- double *delta_height )

- latitude            : Geodetic latitude in radians

- longitude           : Geodetic longitude in radians

- scale_factor        : Grid scale factor

- num_cols            : Number of columns in grid

- num_rows            : Number of rows in grid

- height_buffer       : Grid of height adjustments

- delta_height        : Height Adjustment, in meters

### 5.5.3 DECLARATIONS

#### 5.5.3.1	TYPES

Not applicable.

#### 5.5.3.2	CONSTANTS

Not applicable.

#### 5.5.3.3	VARIABLES

Not applicable.

### 5.5.4	DEPENDENCIES

- None.

### ERROR hANDLING

This function returns the following status codes:

GEOID_FILE_OPEN_ERROR		: Geoid file opening error

GEOID_INITIALIZE_ERROR		: Geoid separation database can not initialize

## 5.6	NS_INTERPOLATE

### 5.6.1	DESCRIPTION

This private function returns the height of the geoid above or below the WGS 84 ellipsoid, at the specified geodetic coordinates, using a grid of height adjustments and a natural spline interpolation method.

5.6.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long NS_Interpolate( double latitude,

- double longitude,

- double scale_factor,

- int num_cols,

- int num_rows,

- float height_buffer[],

- double *delta_height )

- latitude            : Geodetic latitude in radians

- longitude           : Geodetic longitude in radians

- scale_factor        : Grid scale factor

- num_cols            : Number of columns in grid

- num_rows            : Number of rows in grid

- height_buffer       : Grid of height adjustments

- delta_height        : Height Adjustment, in meters.

### 5.6.3 DECLARATIONS

#### 5.6.3.1	TYPES

Not applicable.

#### 5.6.3.2	CONSTANTS

Not applicable.

#### 5.6.3.3	VARIABLES

Not applicable.

### 5.6.4	DEPENDENCIES

- None.

### ERROR hANDLING

This function returns the following status codes:

GEOID_FILE_OPEN_ERROR		: Geoid file opening error

GEOID_INITIALIZE_ERROR		: Geoid separation database can not initialize

## 5.7	Convert_ELLIPSOID_To_GEOID_HEIGHT

### 5.7.1	DESCRIPTION

This function converts the specified WGS 84 ellipsoid height at the specified geodetic coordinates to the equivalent geoid height, using the EGM 96 gravity model with a 15-minute resolution grid and a binlinear interpolation method.

### 5.7.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_Ellipsoid_To_Geoid_Height ( double Latitude,

- double Longitude,

- double Ellipsoid_Height,

- double *Geoid_Height )

- Latitude	Latitude in radians (input),

- Longitude	Longitude in radians (input),

- Ellipsoid_Height	Height above the WGS 84 ellipsoid in meters (input),

- Geoid_Height	Height above the EGM 96 geoid in meters (output).

Example:

- status = Convert_Ellipsoid_To_Geoid_Height (Latitude, Longitude, Ellipsoid_Height, Geoid_Height)

Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Ellipsoid_Height:	0.0

Outputs:

- Geoid_Height:	43.0

### 5.7.3	DECLARATIONS

#### 5.7.3.1	TYPES

Not applicable.

#### 5.7.3.2	CONSTANTS

Not applicable.

#### 5.7.3.3	VARIABLES

double delta_height	height between the ellipsoid and Geoid

long error_code	holds error code when generated

### 5.7.4	DEPENDENCIES

Blinear_Interpolate – Used to calculate the delta_height.

### 5.7.5	ERROR HANDLING

This function returns the following status codes:

GEOID_LAT_ERROR	: Latitude out of valid range (-90 to 90 degrees)

GEOID_LON_ERROR	: Longitude out of valid range (-180 to 360 degrees)

## 5.8	Convert_ELLIPSOID_To_MSL_EGM96_VG_NS_HEIGHT

### 5.8.1	DESCRIPTION

This function converts the specified WGS 84 ellipsoid height at the specified geodetic coordinates to the equivalent geoid height, using the EGM 96 gravity model with a variable resolution grid and a natural spline interpolation method.

### 5.8.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_Ellipsoid_To_MSL_EGM96_VG_NS_Height ( double Latitude,

- double Longitude,

- double Ellipsoid_Height,

- double *Geoid_Height )

- Latitude	Latitude in radians (input),

- Longitude	Longitude in radians (input),

- Ellipsoid_Height	Height above the WGS 84 ellipsoid in meters (input),

- Geoid_Height	Height above the EGM 96 geoid in meters (output).

Example:

- status = Convert_Ellipsoid_To_MSL_EGM96_VG_NS_Height (Latitude, Longitude,   Ellipsoid_Height, Geoid_Height)

Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Ellipsoid_Height:         0.0

Outputs:

- Geoid_Height:	43.0

### 5.8.3	DECLARATIONS

#### 5.8.3.1	TYPES

Not applicable.

#### 5.8.3.2	CONSTANTS

Not applicable.

#### 5.8.3.3	VARIABLES

double delta_height	height between the ellipsoid and Geoid

long error_code	holds error code when generated

### 5.8.4	DEPENDENCIES

NS_Interpolate – Used to calculate the delta_height.

### 5.8.5	ERROR HANDLING

This function returns the following status codes:

GEOID_LAT_ERROR	: Latitude out of valid range (-90 to 90 degrees)

GEOID_LON_ERROR	: Longitude out of valid range (-180 to 360 degrees)

## 5.9	Convert_ELLIPSOID_To_MSL_EGM84_10D_BL_HEIGHT

### 5.9.1	DESCRIPTION

This function converts the specified WGS 84 ellipsoid height at the specified geodetic coordinates to the equivalent geoid height, using the EGM 84 gravity model with a 10-degree grid and a bilinear interpolation method.

### 5.9.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_Ellipsoid_To_MSL_EGM84_10D_BL_Height ( double Latitude,

- double Longitude,

- double Ellipsoid_Height,

- double *Geoid_Height )

- Latitude	Latitude in radians (input),

- Longitude	Longitude in radians (input),

- Ellipsoid_Height	Height above the WGS 84 ellipsoid in meters (input),

- Geoid_Height	Height above the EGM 84 geoid in meters (output).

Example:

- status = Convert_Ellipsoid_To_MSL_EGM84_10D_BL_Height (Latitude, Longitude, Ellipsoid_Height, Geoid_Height)

Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Ellipsoid_Height:	0.0

Outputs:

- Geoid_Height:	43.0

### 5.9.3	DECLARATIONS

#### 5.9.3.1	TYPES

Not applicable.

#### 5.9.3.2	CONSTANTS

Not applicable.

#### 5.9.3.3	VARIABLES

double delta_height	height between the ellipsoid and Geoid

long error_code	holds error code when generated

### 5.9.4	DEPENDENCIES

BL_Interpolate – Used to calculate the delta_height.

### 5.9.5	ERROR HANDLING

This function returns the following status codes:

GEOID_LAT_ERROR	: Latitude out of valid range (-90 to 90 degrees)

GEOID_LON_ERROR	: Longitude out of valid range (-180 to 360 degrees)

## 5.10	Convert_ELLIPSOID_To_MSL_EGM84_10D_NS_HEIGHT

### 5.10.1	DESCRIPTION

This function converts the specified WGS 84 ellipsoid height at the specified geodetic coordinates to the equivalent geoid height, using the EGM 84 gravity model with a 10-degree grid and a natural spline interpolation method.

### 5.10.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_Ellipsoid_To_MSL_EGM84_10D_NS_Height ( double Latitude,

- double Longitude,

- double Ellipsoid_Height,

- double *Geoid_Height )

- Latitude	Latitude in radians (input),

- Longitude	Longitude in radians (input),

- Ellipsoid_Height	Height above the WGS 84 ellipsoid in meters (input),

- Geoid_Height	Height above the EGM 84 geoid in meters (output).

Example:

- status = Convert_Ellipsoid_To_MSL_EGM84_10D_NS_Height (Latitude, Longitude, Ellipsoid_Height, Geoid_Height)

Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Ellipsoid_Height:	0.0

Outputs:

- Geoid_Height:	43.0

### 5.10.3	DECLARATIONS

#### 5.10.3.1	TYPES

Not applicable.

#### 5.10.3.2	CONSTANTS

Not applicable.

#### 5.10.3.3	VARIABLES

double delta_height	height between the ellipsoid and Geoid

long error_code	holds error code when generated

### 5.10.4	DEPENDENCIES

NS_Interpolate – Used to calculate the delta_height.

### 5.10.5	ERROR HANDLING

This function returns the following status codes:

GEOID_LAT_ERROR	: Latitude out of valid range (-90 to 90 degrees)

GEOID_LON_ERROR	: Longitude out of valid range (-180 to 360 degrees)

## 5.11	Convert_GEOID_TO_ELLIPSOID_hEIGHT

### 5.11.1	DESCRIPTION

This function converts the specified geoid height at the specified geodetic coordinates to the equivalent WGS 84 ellipsoid height, using the EGM 96 gravity model with a 15-minute grid and a bilinear interpolation method.

5.11.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_Geoid_To_Ellipsoid_Height ( double Latitude,

- double Longitude,

- double Geoid_Height,

- double *Ellipsoid_Height )

- Latitude	Latitude in radians (input),

- Longitude	Longitude in radians (input),

- Geoid_Height	Height above the EGM 96 geoid in meters (output).

- Ellipsoid_Height	Height above the WGS 84 ellipsoid in meters (input),

Example:

- status = Convert_Geoid_To_Ellipsoid_Height (Latitude, Longitude, Geoid_Height, *Ellipsoid_Height)

Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Geoid_Height:	43.0

Outputs:

- Ellipsoid_Height:	0.0

### 5.11.3	DECLARATIONS

#### 5.11.3.1TYPES

Not applicable.

#### 5.11.3.2CONSTANTS

Not applicable.

#### 5.11.3.3VARIABLES

double 	delta_height	height between the ellipsoid and Geoid

long error_code	holds error code when generated

### 5.11.4	DEPENDENCIES

BL_Interpolate – Used to calculate the delta_height.

### 5.11.5	ERROR HANDLING

This function returns the following status codes:

GEOID_LAT_ERROR	: Latitude out of valid range (-90 to 90 degrees)

GEOID_LON_ERROR	: Longitude out of valid range (-180 to 360 degrees)

## 5.12	Convert_MSL_EGM96_VG_NS_TO_ELLIPSOID_hEIGHT

### 5.12.1	DESCRIPTION

This function converts the specified geoid height at the specified geodetic coordinates to the equivalent WGS 84 ellipsoid height, using the EGM 96 gravity model with a variable resolution grid and a natural spline interpolation method.

5.12.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_MSL_EGM96_VG_NS_To_Ellipsoid_Height ( double Latitude,

- double Longitude,

- double Geoid_Height,

- double *Ellipsoid_Height )

- Latitude	Latitude in radians (input),

- Longitude	Longitude in radians (input),

- Geoid_Height	Height above the EGM 96 geoid in meters (output).

- Ellipsoid_Height	Height above the WGS 84 ellipsoid in meters (input),

Example:

- status = Convert_MSL_EGM96_VG_NS_To_Ellipsoid_Height (Latitude, Longitude, Geoid_Height, *Ellipsoid_Height)

Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Geoid_Height:	43.0

Outputs:

- Ellipsoid_Height:	0.0

### 5.12.3	DECLARATIONS

#### 5.12.3.1TYPES

Not applicable.

#### 5.12.3.2CONSTANTS

Not applicable.

#### 5.12.3.3VARIABLES

double 	delta_height	height between the ellipsoid and Geoid

long error_code	holds error code when generated

### 5.12.4	DEPENDENCIES

NS_Interpolate – Used to calculate the delta_height.

### 5.12.5	ERROR HANDLING

This function returns the following status codes:

GEOID_LAT_ERROR	: Latitude out of valid range (-90 to 90 degrees)

GEOID_LON_ERROR	: Longitude out of valid range (-180 to 360 degrees)

## 5.13	Convert_MSL_EGM84_10D_BL_TO_ELLIPSOID_hEIGHT

### 5.13.1	DESCRIPTION

This function converts the specified geoid height at the specified geodetic coordinates to the equivalent WGS 84 ellipsoid height, using the EGM 84 gravity model with a 10-degree grid and a bilinear interpolation method.

5.13.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_MSL_EGM84_10D_BL_To_Ellipsoid_Height ( double Latitude,

- double Longitude,

- double Geoid_Height,

- double *Ellipsoid_Height )

- Latitude	Latitude in radians (input),

- Longitude	Longitude in radians (input),

- Geoid_Height	Height above the EGM 84 geoid in meters (output).

- Ellipsoid_Height	Height above the WGS 84 ellipsoid in meters (input),

Example:

- status = Convert_MSL_EGM84_10D_BL_To_Ellipsoid_Height (Latitude, Longitude, Geoid_Height, *Ellipsoid_Height)

Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Geoid_Height:	43.0

Outputs:

- Ellipsoid_Height:	0.0

### 5.13.3	DECLARATIONS

#### 5.13.3.1TYPES

Not applicable.

#### 5.13.3.2CONSTANTS

Not applicable.

#### 5.13.3.3VARIABLES

double 	delta_height	height between the ellipsoid and Geoid

long error_code	holds error code when generated

### 5.13.4	DEPENDENCIES

BL_Interpolate – Used to calculate the delta_height.

### 5.13.5	ERROR HANDLING

This function returns the following status codes:

GEOID_LAT_ERROR	: Latitude out of valid range (-90 to 90 degrees)

GEOID_LON_ERROR	: Longitude out of valid range (-180 to 360 degrees)

## 5.14	Convert_MSL_EGM84_10D_NS_TO_ELLIPSOID_hEIGHT

### 5.14.1	DESCRIPTION

This function converts the specified geoid height at the specified geodetic coordinates to the equivalent WGS 84 ellipsoid height, using the EGM 84 gravity model with a 10-degree grid and a natural spline interpolation method.

5.14.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_MSL_EGM84_10D_NS_To_Ellipsoid_Height ( double Latitude,

- double Longitude,

- double Geoid_Height,

- double *Ellipsoid_Height )

- Latitude	Latitude in radians (input),

- Longitude	Longitude in radians (input),

- Geoid_Height	Height above the EGM 84 geoid in meters (output).

- Ellipsoid_Height	Height above the WGS 84 ellipsoid in meters (input),

Example:

- status = Convert_MSL_EGM84_10D_NS_To_Ellipsoid_Height (Latitude, Longitude, Geoid_Height, *Ellipsoid_Height)

Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Geoid_Height:	43.0

Outputs:

- Ellipsoid_Height:	0.0

### 5.14.3	DECLARATIONS

#### 5.14.3.1TYPES

Not applicable.

#### 5.14.3.2CONSTANTS

Not applicable.

#### 5.14.3.3VARIABLES

double 	delta_height	height between the ellipsoid and Geoid

long error_code	holds error code when generated

### 5.14.4	DEPENDENCIES

NS_Interpolate – Used to calculate the delta_height.

### 5.14.5	ERROR HANDLING

This function returns the following status codes:

GEOID_LAT_ERROR	: Latitude out of valid range (-90 to 90 degrees)

GEOID_LON_ERROR	: Longitude out of valid range (-180 to 360 degrees)

# APPENDIX A  STRUCTURE/DEPENDENCY DIAGRAMS

This component consists of a single compilation unit and depends only on the ANSI C standard math library.

# APPENDIX B  DEFINITIONS/GLOSSARY

Coordinate – Linear or angular quantities that designate the position that a point occupies in a given reference frame or system.  Also used as a general term to designate the particular kind of reference frame or system, such as Cartesian or spherical coordinates.

Ellipsoid – The surface generated by an ellipse rotating about one of its axes and often used to represent the surface of the Earth.

Ellipsoid Height – The height as referenced relative to the Earth’s ellipsoid surface at a given geodetic position.  Positions above the ellipsoid have positive values and those below have negative values.

Geodetic Coordinates – The quantities of latitude and longitude that define the position of a point on the surface of the earth with respect to the reference ellipsoid.  Also, imprecisely called geographic coordinates.

Geodetic Latitude – The angle between the plane of the equator and the normal to the ellipsoid through the computation point.  Geodetic latitude is positive north of the equator and negative south of the equator.

Geodetic Longitude – The angle between the plane of a meridian and the plane of the prime meridian.  A longitude can be measured from the angle formed between the local and prime meridians at the pole of rotation of the reference ellipsoid, or by the arc along the equator intercepted by these meridians.

Geoid – A theoretical surface over the Earth representing sea level that ungulates due to variations in the Earth’s mass distribution, and is everywhere perpendicular to the direction of gravity.

Geoid Height – The height as referenced relative to the Earth’s Geoid surface at a given geodetic position.  Positions above the Geoid have positive values and those below have negative values.

Meridian – A north-south reference line, particularly a great circle through the geographical poles of the earth, from which longitudes and azimuths are determined; or the intersection of a plane forming a great circle that contains both geographic poles of the earth, and the ellipsoid.

Parallel – A line on the earth, or a representation thereof, that represents the same latitude at every point.

# APPENDIX C  REFERENCES

(1)  Topographic Engineering Center, TEC-SR-7,.Handbook for transformation of DATUMS, PROJECTIONS, GRIDS, AND COMMON COORDINATE SYSTEMS, January 1996.