/*!

\page projection_algos_page Projection Algorithms

\n Projection algorithms allow to define the mesh of a geometrical

object by the projection of another already meshed geometrical object.

<b>Projection 1D</b> algorithm allows to define the mesh of an edge

(or group of edges)

by the projection of another already meshed edge (or group of edges).

\n To apply this algorithm select the edge to be meshed (indicated in

the field \b Geometry of <b>Create mesh</b> dialog box),

<b>Projection1D</b> in the list of 1D algorithms and click the

<em>"Add Hypothesis"</em> button.

The following dialog box will appear:

\image html projection_1d.png

In this menu you can define the \b Name of the algorithm, the already

meshed source \b Edge and the \b Mesh (optional, use it if there are several

different meshes on the same edge). It could also be necessary to

define the orientation of edges, which is done by indicating the

<b>Source Vertex</b> being the first point of the Source Edge and the

<b>Target Vertex</b> being the first point of the created \b Edge. For

a group of edges, <b>Source</b> and <b>Target</b> vertices should be

shared by only one edge of the group. If <b>Source</b> and

<b>Target</b> vertices are specified, the elements of the group must be ajacent.

\n <b>Projection 2D</b> algorithm allows to define the mesh of a face

(or group of faces) by the

projection of another already meshed face (or group of faces). This

algorithm works only

if all edges of the target face have been meshed as 1D Projections of

the edges of the source face.

To apply this algorithm select the face to be meshed (indicated in the

field \b Geometry of <b>Create mesh</b> dialog box), <b>Projection

2D</b> in the list

of 2D algorithms and click the <em>"Add Hypothesis"</em> button. The

following dialog box will appear:

\image html projection_2d.png

In this menu you can define the \b Name of the algorithm, the already

meshed source \b Face and the \b Mesh (optional, use it if there are several

different meshes on the same face). It could also be necessary to

define the orientation of mesh on the face, which is done by

indicating two <b>Source Vertices</b>, which belong to the same edge of the

source face, and two <b>Target Vertices</b>, which belong to the same edge of

the created \b Face.

\n <b>Projection 3D</b> algorithm allows to define the mesh of a shape by

the projection of another already meshed shape.  This algorithm works

only if all faces and edges of the target face have been meshed as 1D

Projections of the faces and edges of the source face. Another

limitation is that this algorithm currently works only on boxes.

To apply this algorithm select the solid to be meshed (indicated in

the field \b Geometry of <b>Create mesh</b> dialog box), <b>Projection

3D</b> in the list of 3D algorithms and click the  button. The

following dialog box will appear:

\image html projection_3d.png

In this menu you can define the \b Name of the algorithm, the already

meshed source <b>3D shape</b> and the \b Mesh (optional, use it if there are

several different meshes on the same shape). It could also be

necessary to define the orientation of mesh on the shape, which is

done by indicating two <b>Source Vertices</b>, which belong to the same edge

of the source <b>3D Shape</b>, and two <b>Target Vertices</b>, which belong to the

same edge of the source <b>3D Shape</b>.

<br><b>See Also</b> a sample TUI Script of a

\ref tui_projection "Projection Algorithms".

*/